The Pro/Engineer to BRL-CAD conversion is a two step process.

First, in Pro/Engineer, you must select "Proe-brl" under the "File"

menu.  You will be prompted for an output file name, a starting region

ident number, a maximum allowed error for the facetization

approximation, and an angle control. The recomended value for the

angle control is 0.5.  After all prompts have been satisfied, the

currently displayed model will be output to a file in BRL-CAD ACSII

format (actually a Tcl script). The output file can then be converted

to BRL-CAD by using "asc2g" or by sourcing the file in MGED using the

"source" command.

To inform Pro/Engineer about the Pro/Toolkit code that

produces ASCII file, you need to create a "protk.dat" file as

follows:

line 1:name proe-brl

line 2:startup spawn

line 3:exec_file 'full path to wherever you install the proe-brl executable'

line 4:text_dir 'full path to the "text" directory'

line 5:revision 2001

line 5:description Convert currently displayed model to BRL-CAD

line 7:end

See "protk.dat" in this directory for an example. The

exec_file must inlcude the name of the executable file, and the

text_dir must include the name of the "text" directory. In the "text"

directory Pro/Engineer will look for a subdirectory named "usascii"

for a file named "usermsg.txt". The "text" directory in this directory

should be used or at least copied verbatim to where you want to use it.

The "protk.dat" file must be located where Pro/Engineer will look for it.

Pro/Engineer will look in your current directory and in Pro/Engineer's "text"

directory (as well as a few other places).

If the "protk.dat" file, the "proe-brl" executable, and the

"usermsg.txt" files are installed properly, you will see the message

"Installation of Proe-BRL converter succeeded" in the Pro/Engineer

message window when Pro/E starts.